#

File: pioneer.info,  Node: Top,  Prev: (philips.info),  Next: (salora.info),  Up: (remotes.info)

******************************************************************************

PIONEER

******************************************************************************

* Menu:

* Technical info::

* Amp::

* Tape::

* CD::

* Tuner::

* Eq::

* Timer::

* Additional info::

#

File: pioneer.info,  Node: Technical info,  Prev: Top,  Next: Amp,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            32 bits

Carrier:                39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+

Header:      |                                |

+                                +----------------+..

16T                       8T

8.8ms                     4.4ms

+--+

1 is coded:  |  |

..+  +------+..

T   3T

+--+

0 is coded:  |  |

..+  +--+..

T  T

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

+--------------------------------+        +--+

Stop code:    |                                |        |  |

+                                +--------+  +

16T                  4T     T

8.8ms                2.2ms

T = 550us

Space between transmissions: 95000us between the headers of the codes.

To use the code (not with the remote package):

GROUP * 10000h + (Code * 100h) + (Code XOR FFh)

For more information please read the documentation.

#

File: pioneer.info,  Node: Amp,  Prev: Technical info,  Next: Amp,  Up: Top

******************************************************************************

AMPLIFIER

******************************************************************************

GROUP:  #A55A

Code    Function

----    --------

#38C7   Power

#50AF   Vol +

#D02F   Vol -

#31CE   Dat/Video

#30CF   Phono

#59A6   CD direct

#7A85   Surround

#

File: pioneer.info,  Node: Tape,  Prev: Amp,  Next: CD,  Up: Top

******************************************************************************

TAPE

******************************************************************************

GROUP:  #659A

Deck a  Deck b

Code    Code    Function

----    ----    --------

#5AA5   #E817   Play

#9A65   #A857   Reverse Play

#DA25   #6897   Stop

#6A95   #8877   Rew

#EA15   #08F7   FF

#18E7   Pause

#28D7   Rec

#

File: pioneer.info,  Node: CD,  Prev: Tape,  Next: Tuner,  Up: Top

******************************************************************************

CD

******************************************************************************

GROUP:  #45BA

Code    Function

----    --------

#E817   Play

#6897   Stop

#8877   Rew

#08F7   FF

#18E7   Pause

#B847   Disc

#

File: pioneer.info,  Node: Tuner,  Prev: CD,  Next: Eq,  Up: Top

******************************************************************************

TUNER

******************************************************************************

GROUP:  #25DA

Code    Function

----    --------

#08F7   Station +

#8877   Station -

#

File: pioneer.info,  Node: Eq,  Prev: Tuner,  Next: Timer,  Up: Top

******************************************************************************

EQ

******************************************************************************

GROUP:  #05FA

Code    Function

----    --------

#BC43   On/Off

#0CF3   Select

#

File: pioneer.info,  Node: Timer,  Prev: Eq,  Next: Additional info,  Up: Top

******************************************************************************

TIMER

******************************************************************************

GROUP:  #659A

Code    Function

----    --------

#9867   Sleep

#738C   Wake up

#B34C   Snooze

#

File: pioneer.info,  Node: Additinoal info,  Prev: Timer,  Up: Top

******************************************************************************

ADDITIONAL INFO

******************************************************************************

The pioneer receiver (amp/tuner) uses #A55A0000 and #25DA0000, but

not the amplifier group number you had listed in the mail you sent me.

PIONEER

0       2       8       A

0       (10)    (+10)   (ch>)   -

1       (8)     sleep   -       -

2       (4)     -       -       video select

3       -       cd      power   -

4       (2)     -       mute    -

5       vol>    balance on      -

6       (6)     (A-B)   -       (fr>)

7       vcr2(am) tape   (mono)  -

8       (1)     -       (<ch)   mute on

9       (9)     bal>    -       -

A       (5)     -       -       select

B       vdp(fm) phono   -       -

C       (3)     -       (am/fm) -

D       <vol    <bal    off     -

E       (7)     tuner   -       (<fr)

F       vcr1    -       -       -

where:  values in parentheses are for group #25DA0000

and other values are for group #A55A0000.

Tom Boldt

che043sa@sail.uwaterloo.ca

(519) 746-5163