#

File: oldpana.info,  Node: Top,  Prev: (nec.info),  Next: (oldtech.info),  Up: (remotes.info)

******************************************************************************

OLDER PANASONIC

******************************************************************************

* Menu:

* Technical info::

* Video::

* TV::

* Portable::

#

File: oldpana.info,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:    22 bits

Carrier:        32kHz

+----------------+

Header:   |                |

+                +------------+..

8T             6T

+----+

1 is coded:  |    |

..+    +------------+..

2T      6T

+----+

0 is coded:  |    |

..+    +----+..

2T   2T

T=420 us to approx 424 us (US of A and Canada)

T=454 us to approx 460 us (Europe and others)

Remote control transmitter data code (MN6030)

Head pulse  Custom code  Data code  Inverse custom code  Inverse data code

(8T high,   (5 bits)     (6 bits)   (1<==>0)             (1<==>0)

8T low

This means that the actual data is 11 bits and that an inverted copy of

the data is added to the code.

+----+

|    |

After the last bit a ( ..+    +  ) is transmitted to terminate the last bit.

2T

Followed by 90T low (for sync) = 41130us (Europe) = 37980us (USA)

#

File: oldpana.info,  Node: Video,  Prev: Technical info, Next: TV, Up: Top

******************************************************************************

Video

******************************************************************************

Code    Function

----    --------

#402    vtr

#6C2    timer rec

#202    1

#222    2

#242    3

#262    4

#282    5

#2A2    6

#2C2    7

#2E2    8

#302    9

#322    10/0

#482    -/--

#442    Ch +

#462    Ch -

#142    play

#002    stop

#042    rew

#062    ff

#0C2    pause

#102    rec

#182    still adv

#1E2    slow

#583    prog

#743    next

#0C3    vps

#5A2    clock/counter

#642    memory/repeat/search

#662    counter reset

#6A2    remaining time

#7A2    audio out

#

File: oldpana.info,  Node: TV,  Prev: Video, Next: Portable, Up: Top

******************************************************************************

TV

******************************************************************************

Code    Function

----    --------

#400    On/Off

#200    1

#220    2

#240    3

#260    4

#280    5

#2A0    6

#2C0    7

#2E0    8

#300    9

#320    0

#480    -/--

#420    C

#440    Ch +

#460    Ch -

#160    Mute

#540    TV/AV

#0E0    N

#580    <-|-> -

#560    <-|-> +

#1C0    Color -

#180    Color +

#5C0    Contrast -

#500    Contrast +

#1E0    Brightness -

#0C0    Brightness +

#380    Treble -

#4E0    Treble +

#3A0    Bass -

#5E0    Bass +

#740    TV

#760    TEXT

#3E0    Alarm

#780    Mix

#380    Reveal

#4E0    List Store

#5E0    Index

#680    Hold

#6E0    Time text

#6A0    Display cancel

#720    F/T/B

#3A0    List/F. Text

#340    Red

#360    Green

#6C0    Yellow

#7C0    Blue

#

File: oldpana.info,  Node: Portable,  Prev: TV, Up: Top

******************************************************************************

PORTABLE

******************************************************************************

Code for: Panasonic portable stereo component Model RX-DS660

ON    = 2093EDh

TAPE  = 36912Dh

TUNR  = 37910Dh

CD    = 35914Dh

AUX   = 3890EDh

VOL-  = 25934Dh

VOL+  = 24936Dh

1     = 16952Dh

2     = 17950Dh

3     = 1894EDh

4     = 1994CDh

5     = 1A94ADh

6     = 1B948Dh

7     = 1C946Dh

8     = 1D944Dh

9     = 1E942Dh

10    = 1095EDh  <- "0" KEY

10+   = 15954Dh  <- "." KEY

_TAPE_

<<    = 297ADh

>>    = 3978Dh

REC   = 896EDh

<     = 996CDh

>     = A96ADh

STOP  = 97EDh

_CD_

2D924D  REP

2B928D  <<_<  (SKIP/SEARCH)

2A92AD  >>_>  (SKIP/SEARCH)

||    = 2992CDh  ||

PLAY  = 2C926Dh  PLAY

PROG  = 2F920Dh  PROG

CANCL = 3091EDh  CANCL

With best regards,

Enrique Chavez                     echavez@as.arizona.edu